1) The gaging software can be found at: www.promeasure.com/gagexpress3.18b1. It is also on each computer located in the Public Documents Directory. The software is self-extracting. Load GageXpress and the Solartron Drivers which are included.

The software will run for 30 days. To authorize the software please forward the serial number, purchase order and the 8 digit alpha numeric code. The will be generated by going to About in either Profile or PRO. Then choose Keys and select the Purchase button.

2) The Drivers Directory can be found at: www.promeasure.com/proxnetdrivers. It is also on each computer located in the Public Documents Directory.

Not that the drivers do not autoload. The USB for the ProXnet should be connected to the computer. Use Device Manager to manually load the drivers.

3) The program can be backed up from the existing computer by going to PROFILE and selecting "LOAD and BACKUP APPLICATION." Choose backup and then select where the file should be located. By default the program will be stored in the public document directory at: Sterling ProMeasure Systems>GageXpress>Backup.

The last application back up can be also found in the public document directory at: Sterling ProMeasure Systems>GageXpress>Backup.

4) Load the application backup by going to PROFILE and selecting "LOAD and BACKUP APPLICATION." Choose load and then locate the zip file from the backup procedure. GageXpress will load the program.

5) The ProXnet drivers may have to be initialized. In the directory www.promeasure.com/proxnetdrivers there is a document, Instructions - ProXnet BUS, that will review the PC Connection for the IMBus.

6) When the gage program is opened in PRO, the ProX drivers will be recognized. The application program will then open and be functional.